Lear 25 Amplified Checklist
                              START

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Batteries | ON/VOLTAGE CHECKED | 24 Lead-Acid 23 NiCad |
| Parking Brake | SET | 400 PSI Minimum |
| Cabin Door | TWO FORWARD LIGHT OUT |  |
| Standby Gyro Emergency Power | ON |  |
| AC/ Aux Heat | OFF |  |
| Beacon | ON |  |
| Engines | START | 10% Throttle 40% Generator GPU=No Generator until disconnect. No Start #2 until ammeter stabilized |
| Generators and Inverters | ON/CHECKED | DC 28-30 AC Green |
| Current Limiters | CHECKED | Reset Breaker with a Response |
| Radar Avionics | STANDBY/ON |  |
| Gyro Flags | OFF |  |
| Parking Brake | RELEASE |  |

TAXI

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Brakes and Steering | CHECKED |  |
| Flight Controls | CHECKED |  |
| Anti-Skid | ON, LIGHTS OUT |  |
| * Windshield Heat | PURGED/ CHECKED OFF |  |
| Spoilers, Flaps Trim | RETRACTED, LIGHT OUT, SET FOR TAKEOFF |  |
| Flight and Nav Instruments | SET AND CHECKED |  |
| Altimeters | SET |  |
| Bugs and V Speeds | SET | Read Aloud |
| Hold 2nd Engine Start |  |  |
| Generators | ON |  |
| Cabin Bleed Air and Pressurization | ON & CHECKED | Switch on Pressurization and set Cruise Altitude |

| TAXI Continued |  |  |
| --- | --- | --- |
| * Stab/Wing Heat | CHECKED |  |
| * Nacelle Heat | CHECKED |  |
| * Thrust Reversers | CHECKED |  |
| Circuit Breakers | CHECKED |  |
| Engine Instruments | NORMAL |  |
| Radar Avionics | ON/STANDBY |  |
| Fuel Panel | CHECKED BALANCED | Tip and Tanks Balanced Within 50 Lbs. |
| Seat Belts Shoulder Harnesses | FASTENED |  |
| Crew Briefing | COMPLETED |  |

LINE UP CHECK

| PNF | PNF | PNF ACTION |
| --- | --- | --- |
| Transponder | ON | TURN ON |
| Anti-Ice | AS REQ’D | CONFIRM |
| Lights | ON | STROBES ON RWY |
| -----------Hold Line----------- |  |  |
| Pitot Heat | ON | TURN ON |
| Stall Warnings | ON | TURN ON |
| Air Ignitions | ON | TURN ON |
| Thrust Reversers | ARM | ARM ON |
| Warning Lights | OUT | CONFIRM |

Power Set - Airspeed Alive - 100 KTS Twice - V1 -
Rotate - Positive Rate:  +30

AFTER TAKEOFF

| PNF | PNF | PNF ACTION |
| --- | --- | --- |
| Landing Gear | UP | CONFIRM |
| Yaw Damper | ON | CONFIRM |
| Flaps | UP | CONFIRM |
| Air Ignitions | OFF | CONFIRM |
| Thrust Reversers | OFF | ARM OFF |
| Landing Lights | OFF | TURN OFF |
| Pressurization | MONTIOR |  |
| Anti-Ice | ON/OFF | AS REQ |
| Angle of Attack | CHECKED | CONFIRM |

CLIMB (FL 180)

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Altimeters | 29.92 | SET |
| Pressurization | MONITOR |  |
| Air Conditioner | OFF | TURN OFF |
| Recognition Light | OFF | TURN OFF |
| Seat Belt No Smoking Sign | AS REQ | AS REQ |

CRUISE

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Defog | IN |  |
| Windshield Heat | ON/OFF |  |
| Pressurization | MONITOR |  |
| Fuel Panel | MONITOR |  |
| Anti-Ice | AS REQ |  |
| Engine Instruments | MONITOR |  |

DESCENT

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Windshield Heat | ON |  |
| Anti-Ice | ON/OFF |  |
| Pressurization | LANDING ELEVATION | SET LANDING ELEVATION |
| Crew Briefing | COMPLETED |  |

APPROACH (FL 180)

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| H Valve Closed | CLOSED |  |
| Defog | OUT |  |
| Altimeters | SET |  |
| Bugs & V -Speeds | SET | Add ½ Fuel Load to 105. Add 1 KT for each 200# Pax or Freight |
| Lights | ON | TURN ON |
| Fuel Balance | CHECKED |  |
| Circuit Breakers | CHECKED |  |
| Hydraulic & Emer Air | CHECKED |  |
| Seat Belt No Smoking | ON |  |
| Radar Altimeter | ON |  |
| Air Conditioner | ON | AS REQ |
| Thrust Reverser | ARMED | PRESS FAULT PRESS TEST THEN ARM |
| FLT & NAV Instruments | SET & CHECKED |  |

BEFORE LANDING

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Spoilers | RETRACTED |  |
| Landing Gear | DOWN |  |
| Landing Lights | ON | TURN ON |
| Hydraulic Pressure | CHECKED |  |
| Anti-Skid | CHECKED-ON |  |
| Parking Brake | OFF |  |
| Flaps | LAND |  |
| Air Ignition | ON | TURN ON |
| Yaw Damper | OFF |  |

AFTER LANDING

| PNF | PF | PNF ACTION |
| --- | --- | --- |
| Thrust Reversers | OFF | TURN OFF |
| Ignition | OFF | TURN OFF |
| Stall Warning | OFF | TURN OFF |
| Landing / Taxi Lights | TAXI OR OFF | TURN ON/OFF |
| Recog / Strobe Lights | OFF | TURN OFF |
| Spoilers/Flaps/Trim | RETRACTED/UP/RESET | RETRACT, UP, RESET |
| Radar and Unnecessary Avionics | OFF | TURN OFF |
| Pitot Heat & Anti-Ice | OFF | TURN OFF |
| Emergency Power | OFF |  |
| Cabin Bleed / H-Valve | OFF | TURN OFF |
| Hydraulic Pressure | CHECKED |  |
| Fuel Transfer | AS REQ |  |

SHUTDOWN

| PNF | PNF | PNF ACTION |
| --- | --- | --- |
| CHOCKS OR PARKING BRAKE | SET |  |
| PANEL SWITCHES AND AVIONICS | OFF | TURN OFF |
| THRUST LEVERS | CUTOFF | CONFIRM |
| CROSS FLOW VALVE | CLOSED | CLOSE |
| FUEL TRANSFER FUSELAGE VALVE | OFF / CLOSED | CLOSE |
| HYDRAULIC PUMP | OFF | CONFIRM |
| INVERTERS AND GENERATORS | OFF | CONFIRM |
| OXYGEN HOSES | DISCONNECT | DISCONNECT |
| BATTERIES | OFF | CONFIRM |
| CONTROLS | LOCK |  |
| ENTRY LIGHTS | OFF | TURN OFF |