Lear 35A SER# 590 Limitations with FC-530 by Value

| Increase Landing Distance S.E | 1.2 | % |
| --- | --- | --- |
| N1 or N2 Overspeed 1 Minute to 103% | 100 | % |
| N1 or N2 Overspeed 5 Sec to 105 | 103 | % |
| Generator Output Ground | 320 | Amps |
| Generator Output Flight | 400 | Amps |
| Max GPU Output @ 28 Volts | 1100 | Amps |
| Pressurization goes to Manual Control | 8,700 | Cabin |
| Emerg. Press. Valves Open directing Engine Bleed into Cabin | 9,500 | Cabin |
| Cabin Altitude Warning Horn | 10,100 | Cabin |
| Cabin Altitude Limiters Actuate | 11,500 | Cabin |
| Pax Mask not intended for prolonged use above | 25,000 | Cabin |
| Crew and Pax O2 Not Approved Above | 40,000 | Cabin |
| Fuel Temp | -54 | Deg. C |
| Fuel Temp for Takeoff other than JP4 or equiv | -29 | Deg. C |
| Use GPU | 0 | Deg. C |
| Anti-Ice On Ground | 4.4 | Deg. C |
| Anti-Ice On Flight | 10 | Deg. C |
| Max Oil Temp | 140 | Deg. C |
| Recommended Turbine Temp | 795 | Deg. C |
| ITT Limits Max Cont | 832 | Deg. C |
| ITT Limits Start | 860 | Deg. C |
| ITT Limits Takeoff | 860 | Deg. C |
| ITT Limits Max Transient | 870 | Deg. C |
| Trim Takeoff Range to 7.6 | 5 | Degrees |
| O2 Masks Deploy, Cabin overhead panel lights on automatically | 14,000 | Feet |
| Intentional stalls with flaps and gear extended not above | 18,000 | Feet |
| O2 Masks must be quick donning above | 25,000 | Feet |
| Max Operating Altitude | 45,000 | Feet |
| A/P Minimum Altitudes Approach | 200 | Feet AGL |
| A/P Minimum Altitudes Cruise | 1000 | Feet AGL |
| Max Imbalance During Fueling | 125 | Gallons |
| When using any AV Fuel in the mix Gal per 100 | 500 | HRS |
| Max Water/Slush | ¾ | Inch |
| Vmcg | 103 | KIAS |
| Vmca Sea Level -18c (0 f) | 110 | KIAS |
| Max Vref | 129 | KIAS |

| Vfe 21-40 | 150 | KIAS |
| --- | --- | --- |
| Vfe 9-20 | 185 | KIAS |
| Vfe 1-8 | 200 | KIAS |
| Vlo | 200 | KIAS |
| Vle | 260 | KIAS |
| Airspeed Red Line /Barber Pole 67-670 | 300 | KIAS |
| Vmo: Up to 8000 Feet | 300 | KIAS |
| Vmo if additional fuselage fuel cannot be transferred (See Max ZWF below) | 325 | KIAS |
| Airspeed Red Line /Barber Pole 018-063 | 350 | KIAS |
| Vmo: 8000 and above | 350 | KIAS |
| Max Tailwind | 10 | KTS |
| Nosewheel Steering Speed and 2 A/S Lights Inop | 10 | KTS |
| Max Nosewheel Steering Speed | 45 | KTS GS |
| Max Tire Limit | 182 | KTS GS |
| Un-transferable Fuel with Gravity Flow | 162 | LBS |
| A/P Fuel Imbalance | 200 | LBS |
| Low Fuel | 450 | LBS |
| Max Bags in Rear | 500 | LBS |
| Minimum Fuel each wing TO or G/A | 600 | LBS |
| Soonest can move fuel to wings (Tip Fuel) | 760 | LBS |
| Max Tip Landing WT | 925 | LBS |
| Aux Hydraulic Pressure | 1250 | LBS |
| Normal Hydraulic Pressure | 1550 | LBS |
| Approx Useable Fuel | 6,238 | LBS |
| BOW SIM | 10,500 | LBS |
| Max Zero Wing and Tip Fuel Weight | 13,500 | LBS |
| Max Zero Wing and Tip Fuel Weight by adding up to 500 lbs additional fuselage fuel. Must be transferred prior to reaching a total wing fuel quantity of 2250 pounds | 14,000 | LBS |
| Max Landing Weight | 15,300 | LBS |
| Max Takeoff | 18,300 | LBS |
| Max Ramp | 18,500 | LBS |
| Max w/o Mach Trim A/P OFF | .69 | Mach |
| Mmo with Mach Trim and Autopilot Disengaged or left Stall Warning Switch Off | .74 | Mach |
| Mmo with any BLE missing | .78 | Mach |
| Mmo | .81 | Mach |
| Aux Hydraulic Pump Minutes on (20 Min off) | 3 | Min |
| Max Water/Slush | 19 | MM |
| Max Pressure Altitude for TO | 10,000 | MSL |
| Without Emerg Press Valves Wear Masks Abv | 41,000 | MSL |
| Max Diff Press | 10 | PSI |
| Halon Bottle @ 70 Degrees F | 600 | PSI |
| Freon Air Cond (No FCN 89-1) Off Above | 18,000 | PSI |
| Freon Air Cond (FCN 89-1) Off Above | 35,000 | PSI |