Lear 35A SER# 234 & 590 Limitations with FC-530 Autopilot Sorted by Subject

| Limitation | Associated Number |
| --- | --- |
| Approved for VFR, IFR, day, night, icing conditions. |  |
| Aux Hydraulic Pump | 3 min on 20 min off |
| Both Stall Warning Switches must be on |  |
| Certified under part 25 |  |
| Crew and Pax O2 Not Approved Above | 40,000 ft. Cabin Alt |
| Do not extend or operate spoilers above Mmo Vmo |  |
| Do not extend spoilers or operate with spoilers deployed at speeds above Vmo Mmo |  |
| Do not extend spoilers with flaps extended while airborne |  |
| Do not take off with Cabin Pressurized |  |
| Engine Sync off for TO, LDG, SE Opns |  |
| Flight Loads Flaps Up / Flaps Down | +3.0 to -1 g (70 degree bank) +2.0 to 0 g (60 degree bank) Pullups, overspeeds, push-overs limited to these values |
| Freon Air Cond (No FCN 89-1) / (FCN 89-1) Off Above | 18,000 / 35,000 |
| Generator Output Flight / Ground | 400 / 320 |
| Hydraulic fluid | MIL - H-5606 |
| Intentional stalls above 18,000 ft. prohibited with flaps and gear extended |  |
| Max GPU Output | 1,100 Amps |
| Max Landing Weight | 15,300 lbs. |
| Max Nosewheel Steering Speed | 45 Kts GS |
| Max Operating Altitude | 45,000 ft |
| Max Pressure Altitude for TO | 10,000 ft. MSL |
| Max Ramp most Aircraft | 18,500 lbs |
| Max Tailwind | 10 KTS |
| Max Takeoff most aircraft | 18,300 lbs. |
| Max Tip Landing WT | 925 lbs. |
| Max Tire Limit | 182 Kts GS |
| Max Water/Slush | ¾” 19mm |
| Max Zero Wing and Tip Fuel Weight | 13,500 lbs. |
| Minimum flight crew is (2) two. |  |
| Minimum Fuel each wing TO or G/A | 600 Lbs. |
| Mmo | .81 Mach |
| Mmo with any BLE missing | .77 Mach |
| Mmo with Mach Trim and Autopilot Disengaged | .74 |
| Mmo with Stick Puller Inoperative (left S.W. switch off) | .74 |
| No Aerobatics including spins are approved |  |
| One yaw damper must be on for all flight conditions except TO, LDG, and rudder trimming |  |
| Pitch Trim runaway training prohibited in fliht |  |
| Seat Belts and Shoulder Harness worn TO & Ldg |  |
| Va (Stall speed with 3.0 g’s/ gross wt or highest speed that full rudder & aileron can be applied without overstress. |  |
| Vfe 1-8 / 9-20 / 21-40 | 200/ 185-200 / 150 KIAS |
| Vle | 260 KIAS |
| Vlo | 200 KIAS |
| Vmca Sea Level -18c (0 f) | 110 KIAS |
| Vmcg | 103 KIAS |
| Vmo: Up to 8000 ft. / 8000 and above | 300 KIAS / 350 KIAS |
| With no CUR LIM or Start Disengage Check every start |  |
| Yaw Damp checks must be completed before each flt. |  |

| Pax Mask intended for use during | Emerg Descent |
| --- | --- |
| Pax Mask will not provide sufficient O2 Above | 34,000 ft Cabin Alt |
| Pax Mask not intended for prolonged use above | 25,000 ft Cabin Alt |
| Without Emerg Press Valves Wear Masks Abv | 41,000 MSL |
| O2 Masks must be quick donning above | 25,000 ft |
| Max Diff Press | 10 psi |
| Windshield and Radome Alcohol grade | 0-M-232 Grade A |
| N1 or N2 Overspeed 5 Sec / 1 Minute is | 103-105% / 100-103% |
| Max Oil Temp | 140 C |
| Oil Type | EMS 53110, Type II |
| ITT Limits Start/Takeoff/Max Cont/Max Transient | 860/860 (5 Min) / 832 / 870 |
| Fuel Temp for Takeoff other than JP4 or equiv | -29c (-20f) |
| Fuel Temp | -54c (-65f) |
| Any fuel in the tanks that does not register on the fuel quantity indicator is considered unusable |  |
| Fuel Anti-Icing either | MIL-I-27686 or MIL-I-85470 |
| EFC must be adjusted to the Specific Gravity of Fuel |  |
| Mixing of fuel types is allowed |  |
| AV Fuel in the mix no Takeoff with Fuel Temp Lower than -54c Restricted below 15,000 feet Both Jet and Standby Pumps must be on and operating AvGas may not be use din excess of | 500 Gal per 100 Hrs |
| Airspeed Red Line /Barber Pole 67-670 and 018-063 | 300 KIAS /350/.81 |
| Airspeed Red Line /Barber Pole 671- and 064 - | 350 KIAS /350/.81 |
| Trim Takeoff Range | 5 - 7.6 degrees |
| Cabin Altimeter Green/Yellow/Red 67-112 / 107, 133 - | 0-8.9/9.2/10 / 0-9.4/9.7/10 |
| Turbine Speed Green/Yellow/Red | 48-100/103 1 Minute/103% |
| Turbine ITT Green/Yellow/Red | 230-832/860/860c |
| N1 Fan Green/Yellow/Red | 24-100/103 1 Minute/103% |
| Oil Pressure Red/Yellow/Green/Yellow/Red | 25/25-38/38-46/46-55/55 3 Min. Max |
| Oil Temperature Green/Yellow/Red | 30-127/140/140 |
| Ammeter Yellow/Red | 320-400/400 |
| DC Voltmeter Red | 32 |
| AC Voltmeter Green | 110-130 |
| Hydraulic Pressure 067-646 Yellow/Green/Red | 0-155/1,250-1,500/1,750 PSI |
| Emergency Air Green/Red - Yellow/Green/Red | 1,800-3,000 - 1,800/3,000/3,000 PSI |
| O2 Pressure Green/Yellow/Red | 1,550-1,850/0-300/2000PSI |
| AUTOPILOT FLIGHT DIRECTOR LIMITATIONS |  |
| AP/FD are approved for CAT I ILS Approaches |  |
| Max Operating Speed is Vmo.Mmo |  |
| In Seat, Belt Fastened |  |
| Pitch and Roll Axis NOT during TO & LDG |  |
| Minimum Altitudes Cruise/Approach | 1,000 agl/200 agl |
| Fuel Imbalance | 200 lbs |
| If Severe Turbulence use Alt Hold and Soft Mode |  |
| No use any axis if fails AP Monitor Check |  |
| Disengage before switching static sources |  |
| For VOR Approaches lower flaps 8 or more |  |
| DRAGE CHUTE |  |
| No Deploy in Flight |  |
| Max Speed 150 Kts |  |
| Max Demo XW 20 Kts |  |
| DEE HOWARD T/R |  |
| Ground Deployment only |  |
| Engine Starts with T/R Prohibited |  |
| No Use for Backing |  |
| Disarm Immediately after T/O remain until Final Appch |  |
| Full Reverse is 50 Kts |  |
| Maximum Fan Speed in Reverse is 75% |  |
| Simultaneous T/R and Drag Chute Prohibited |  |