Lear 35A Big 7 Emergency Procedures

| # Aborted Takeoff | - Thrust Levers IDLE - Wheel Brakes APPLY - Spoilers EXTENDED |
| --- | --- |
| # Cabin Alt 10,000’ Warning (Emergency Descent) | - Crew Oxygen Masks DON & Select 100% - Thrust levers IDLE - Autopilot DISENGAGE - Spoilers EXTEND - Landing Gear Mmo / Vle DOWN - Descend at Mmo / Vle - PASS OXY Valve NORMAL - PASS MASK Valve MAN |
| # Engine Failure Approach | - CWMS DEPRESS AND RELEASE - Thrust Lever Operative Engine INCREASE AS REQ’D - Flaps 20 MAX - Airspeed VREF + 10 MIN |
| # Engine Failure Above V1 | - Rudder & Ailerons AS REQ’D - Accelerate to Vr NOSE ON Runway - Rotate at Vr; Climb at V2 - Positive Rate GEAR UP - Clear of Obstacles V2+30 FLAPS UP |
| # Engine Failure Below V1 | - Thrust Levers IDLE - Wheel Brakes APPLY - Spoilers EXTEND |
| # Engine Fire - Shutdown | - Thrust Lever IDLE UNLESS CRITICAL - If fire or continues more than 15 seconds: a. Thrust Lever CUTOFF - Engine Fire Pull Handle PULL - ARMED Light DEPRESS ONE |
| # Pitch Axis Malfunction | - Control Wheel Master Switch DEPRESS - Attitude Control AS REQ’D - Thrust Levers: If high-speed nose-down attitude IDLE If near stall INCREASE AS REQ’D - Both Stall Warning Switches OFF - Pitch Trim Switch OFF - Autopilot Switch OFF |