Lear 55 Operational Checks
                             SUMMARY

| ANTI-ICE |  |
| --- | --- |
| Left Nac Heat Sw | ON |
| Left Eng Anti-Ice Light | ILLUMINATES BRIEFLY |
| Switch Off |  |
| Right Nac Heat Sw | ON |
| Right Eng Anti-Ice Light | ILLUMINATES BRIEFLY |
| Switch Off |  |
| Stab Wing Heat Switch | ON |
| Stab Wing Heat Light | ON - VERIFY WING TEMP INCREASE |
| AUTOMATIC POWER RESERVE |  |
| Both Thrust Levers | IDLE |
| APR Switch | ARM - Verify ARM Light ON |
| One Thrust Lever | ADVANCE 5%-10% N2 |
| Verify | APR ON Light Illuminated |
| Thrust Lever | IDLE |
| APR Switch | RESET/OFF |
| Verify APR ON & APR ARM Lights | OFF - Both N2 Decrease 1% |
| AUTOPILOT MONITOR |  |
| Autopilot Switch | ON |
| Test Button | PUSH - VERIFY All Lights Illuminate |
| Test and Engage Buttons | PUSH |
| ROLL Light | Verify Out in 6 Seconds w/Tone |
| EMERGENCY PRESSURIZATION |  |
| Left Bleed Air Switch | EMERGENCY |
| EMER PRESS LIGHT | Verify Light on and Airflow |
| Switch to OFF | Verify Light off and Airflow Stops |
| Right Bleed Air Switch | EMERGENCY |
| EMER PRESS LIGHT | Verify Light on and Airflow |
| Switch to OFF | Verify Light off and Airflow Stops |

| MACH MONITOR |  |
| --- | --- |
| Pitch Trim | IN TAKEOFF PIE |
| Mach MON | SELECT ON TEST SWITCH |
| Trim | NOSE UP OR NOSE DOWN |
| Test Switch | PUSH |
| Verify | TRIM STOPS |
| Verify | RED MACH TRIM, AMBER PITCH TRIM Lights ON |
| Test Switch | Release - Lights Extinguish |
| MACH TRIM / MACH TRIM MONITOR |  |
| Mach TRIM | SELECT ON TEST SWITCH |
| Pitch Trim Indicator | WATCH |
| Push Test Button |  |
| Verify | 1 - 3 Seconds Nose Up Trim |
| Verify | TRIM STOPS |
| Verify | RED MACH TRIM LIGHT and WARNING HORN ON |
| Release Test Button | Horn Stops, Light Goes Out |
| STALL WARNING SYSTEM |  |
| Stall Warning Switches | ON |
| Flap Switch | DOWN (SSK55-504) |
| Left Stall Test Switch | Press and Hold |
| Verify | Needle to Red, Momentary Horn |
| Verify | Light, Shaker, Nudger, Pusher |
| Verify | Needle returns Yellow or Green |
| Left Stall Test Switch | Release |
| MSW and Test Switch | Depress Both |
| Verify | Horn, Light, Shaker |
| Verify | No Nudger or Pusher |
| Test Switch | Release |
| Operate Flaps | One complete retraction or extension |
| Verify | Both Stall Margin Indicators make significant shift in position |
| Stall Warning Switches | OFF |

| STICK PULLER / MACH WARNING |  |
| --- | --- |
| Pitch Trim | IN TAKEOFF PIE |
| Left Stall Switch | ON |
| Select MACH on Test Switch | PRESS AND HOLD |
| Verify Activation of | OVERSPEED WARNING HORN and PULLER |
| After one second |  |
| Verify | OVERSPEED WARNING HORN and PULLER STOP and RE-ACTIVATE |
| Release Button | HORN AND PULLER STOP |
| SPOILERONS/ SPOILERS |  |
| Verify | SPOILERS RETRACTED FLAPS FULL DOWN |
| Control Wheel | CENTER and ENGAGE Reset Switch |
| Control Wheel | TURN LEFT Unitl AUG AIL SPOILER MON Light Illuminates |
| Center Wheel | Release and Re-Engage Test Switch |
| Control Wheel | CENTER and ENGAGE Reset Switch |
| Control Wheel | TURN RIGHT Unitl AUG AIL SPOILER MON Light Illuminates |
| Control Wheel | RELEASE then click RESET Switch to Extinguish Light |
| Control Wheel | Full Left and Full Right - Verify NO LIGHT |
| Spoilers | EXTEND - Verify FLASHING SPOILER Light |
| Flaps | RETRACT - Verify STEADY SPOILER Light |
| Spoilers | RETRACT -Verify SPOILER LIGHT OUT |
| AUTO-SPOILERS |  |
| Spoiler Switch | ARM - Verify SPOILER ARM and SPOILER LIGHT |
| Reset Switch | Engage - Spoiler Light OFF |
| Reset Switch | Release - Spoiler Light ON |
| Left Thrust Lever | ADVANCE Slightly - Verify Spoiler Light OFF |
| Left Thrust Lever | RETARD - Verify Spoiler Light ON |
| Repeat for Right |  |
| Spoiler Switch | RETRACT - Verify Spoiler and Spoiler ARM lights OFF |

| THRUST REVERSER |  |
| --- | --- |
| Verify | CB’s IN Light OFF |
| Test Button | Push |
| Verify | BLEED PRESS LIGHTS, HORN, Flashing UNLOCK LIGHTS |
| Perform | Normal Deploy |
| Perform | Emergency Stow |
| Stow Levers | SWITCH to NORMAL |
| Perform | Normal Deploy, Normal Stow, Lights OFF |
| TRIM SPEED MONITOR |  |
| Position Trim Indicator | NOSE UP SIDE OF INDEX |
| Trim Speed Test Switch | PRESS AND HOLD - TRIM NOSE UP -Verify PITCH TRIM LIGHT ON |
| Release Test Switch | LIGHT OFF |
| TRIM SYSTEMS |  |
| Secondary Pitch Trim | SELECT |
| Trim with Secondary | NOSE UP and NOSE DOWN - Verify Trim Motor and Clicker |
| Press CWMS | Verify Stops Trim in Both Directions |
| Select Primary Trim | Control Wheel Trim Switch 4 directions without arming - Verify NO TRIM |
| With Arming Button | Trim 4 directions - Verify TRIM MOTION and CLICKER (pitch only) |
| Press CWMS | Verify Stops Trim in Both Directions |
| Repeat on Right Side |  |
|  | Verify Pitch Trim Speed Changes passing Index |
|  | Verify Rudder Trim Both Directions |
| YAW DAMPERS |  |
| Both Power Buttons | PRESS - Verify ON Lights Illuminated |
| Test Button | PRESS - Verify both ENGAGE lights ON Both Force Indicators RIGHT then LEFT |
| Secondary | ENGAGE - Override with Rudder pedals, disconnect with MSW |
| Primary | ENGAGE - Override with Rudder pedals, disconnect with MSW |