Lear 55 Alphabetical Emergency Procedures

| # Aborted Takeoff | - Thrust Levers IDLE - Wheel Brakes APPLY - Spoilers EXTENDED |
| --- | --- |
| # Cabin Alt 10,000’ Warning (Emergency Descent) | - Crew Oxygen Masks DON - Thrust levers IDLE - Autopilot DISENGAGE - Spoilers EXTEND - Descend at Mmo/Mmo |
| # Cabin Fire Light or Cabin/Cockpit Fire, Smoke or Fumes | - Crew Oxygen Masks DON & SELECT 100% - Smoke Goggles DON - Passenger Oxygen Valve-CHECK,AUTO Mask Drop Valve-MAN - OXY-MIC Switches OXY-MIC - If source is not immediately known - Land as soon as possible If source is known - Extinguish fire or eliminate smoke or fumes If it cannot be verified fire is out - Land as soon as possible If fire is out - Land as soon as practical |
| # Emergency Braking | - Emergency Brake Handle PULL OUT - Emergency Brake Handle PUSH DOWNWARD |
| # Emergency Evacuation | - Stop the aircraft - Parking Brake SET - Thrust levers CUTOFF - If an engine fire is suspected a. Applicable Engine Fire Handle PULL b. ARMED Light DEPRESS ONE c. Other Engine Fire Pull Handle PULL If engine fire is not suspected: a. Both Engine Fire Handles PULL - Batteries OFF |
| # Engine Failure During Approach | - Control Wheel Master Switch DEPRESS AND RELEASE - Thrust Lever (operative engine) INCREASE AS REQ’D - Flaps 20 - Airspeed VREF + 10 |
| # Engine Failure During Takeoff Above V1 | - Rudder & Ailerons AS REQ’D - Accelerate to Vr Keep nose wheel on Runway - Rotate at Vr; Climb at V2 - Positive Rate GEAR UP - Clear of Obstacles V2+30 FLAPS UP |
| # Engine Failure During Takeoff Below V1 | - Thrust Levers IDLE - Wheel Brakes APPLY - Spoilers EXTEND (T/R or D/C Deploy if Necessary) |
| # Engine Fire - Shutdown | - Thrust Lever IDLE UNLESS CRITICAL THRUST SITUATION - If fire continues more than 15 seconds or there are other indications of fire: a. Thrust Lever CUTOFF - Engine Fire Pull Handle PULL - ARMED Light DEPRESS ONE |

| # Immediate Engine Airstart | - Thrust Lever IDLE - Ignition ON - Standby Pump ON |
| --- | --- |
| # Overspeed Recovery - Overspeed Warning Horn | - Thrust Levers IDLE - Autopilot DISENGAGE - Identify Aircraft Pitch and Roll Attitude - Level Wings - Elevator and Pitch Trim NOSE UP AS REQ’D If Mach or Airspeed is severe or if pitch and/or roll attitude is extreme or unknown: - Landing Gear DOWN, DO NOT RETRACT |
| # Pitch Axis Malfunction | - Control Wheel Master Switch DEPRESS AND HOLD - Attitude Control AS REQ’D - Thrust Levers: If high-speed nose-down attitude IDLE If near stall INCREASE AS REQ’D - Both Stall Warning Switches OFF - Pitch Trim Switch OFF - Autopilot Switch OFF |
| # Roll or Yaw Axis Malfunction | - Control Wheel Master Switch DEPRESS - Attitude Control AS REQ’D If control force continues - Airspeed REDUCE - Affected Axis Trim CB - ROLL or YAW TRIM (pilot’s ESS bus) PULL |
| # Stall Warning Activates | - Lower Pitch Attitude to reduce angle of attack - Thrust Levers TAKEOFF POWER - Level the wings - Accelerate out of the stall condition |
| # Thrust Reverser - Deploy During Takeoff | - Emer Stow Switch EMER - Throttle IDLE - Positive Rate of Climb GEAR UP - Clear of Obstacles V2+10 FLAPS UP - Maximum Airspeed (until stowed) 125 KIAS |
| # Thrust Reverser Deployment During Takeoff Above V1 | - Rudder and Ailerons AS REQ’D - Thrust Lever (affected engine) IDLE - Emer Stow Switch EMER STOW - Accelerate to Vr Keep nose wheel on runway - Rotate at Vr Climb at V2 - Positive Rate of Climb Established GEAR UP - Clear of Obstacles ACCELERATE TO V2+30, FLAPS UP |
| # Thrust Reverser Deployment During Takeoff Below V1 | - Thrust Levers IDLE - Wheel Brakes APPLY - Spoilers EXTEND |